INDIAN OIL CORPORATION LIMITED
ANNUAL STATUTORY INSPECTION & FITNESS-FOR-SERVICE APPROVAL NOTE (2025)
REFERENCE: PSU/IOCL/UNIT3/2025/112  •  DATE: 18-SEP-2025

| Plant / Refinery Unit: | Paradip Refinery • CDU-3 Crude Pre-Heat Train |
| --- | --- |
| Piping System Tag: | 12"-P-1042-CS-150 (Unit 3 Line 1042) |
| Governing Code: | ASME B31.3 Process Piping / API Standard 570 |
| Previous Approval Status: | APPROVED WITH MANDATORY 12-MONTH RE-SURVEY (SEP 2026) |

1. EXECUTIVE SUMMARY & 2025 FITNESS-FOR-SERVICE VERDICT
During the scheduled statutory turnaround inspection on 16-Sep-2025, ultrasonic thickness gauging was performed across 5 Condition Monitoring Locations (CMLs) on crude line 12"-P-1042-CS-150. The minimum recorded wall thickness was 7.80 mm at CML-03 (governing lower quadrant). The calculated ASME B31.3 statutory minimum required wall thickness is 5.62 mm, demonstrating a compliant structural margin of +2.18 mm and an estimated remaining life of 7.4 years at an evaluated corrosion rate of 0.22 mm/year.

2. STATUTORY APPROVAL CONDITIONS & RE-INSPECTION MANDATE
Continued operation is formally authorized for 12 calendar months subject to the following statutory stipulations:
• Operating pressure shall not exceed 400 psig at 350°F.
• Ultrasonic thickness re-gauging must be conducted no later than September 2026.
• If measured thickness at CML-03 falls below 7.00 mm, comprehensive fitness-for-service re-evaluation per API 570 Section 7 is mandatory prior to permit renewal.

FORMALLY APPROVED BY:

Er. R. K. Sharma
Superintending Engineer (Inspection & NDT)
Indian Oil Corporation Ltd, Paradip Refinery